🌐 CCNA Lab 11 – IPv6 Configuration with DHCPv6 and Basic Router Security

📌 Lab Title

IPv6 Addressing, DHCPv6 Configuration, SSH Setup, and Basic Router Security

🎯 Lab Objective

The objective of this lab was to configure:

✅ IPv6 Routing

✅ IPv6 Address Assignment

✅ DHCPv6 Server

✅ DHCPv6 Address Pool

✅ Domain Name Configuration

✅ SSH Version 2

✅ Local User Authentication

✅ Console Security

✅ Basic Router Security

✅ IPv6 Connectivity Between Router and Client

📖 Introduction to IPv6

What is IPv6?

IPv6 (Internet Protocol Version 6) is the latest version of the Internet Protocol designed to replace IPv4.

IPv4 Example:

192.168.1.1

IPv6 Example:

2026::1

Why IPv6?

IPv4 provides approximately:

4.3 Billion Addresses

IPv6 provides:

340 Undecillion Addresses

(Practically unlimited)

Benefits of IPv6

Larger Address Space

128-bit Address

Better Security

Supports IPSec.

Auto Configuration

Devices can automatically obtain addresses.

Efficient Routing

Improved packet processing.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco Router | 1 |
| Switch | 1 |
| PC | 1 |

📊 Network Topology

PC1
       |
       |
    Switch
       |
       |
      R1

📋 IPv6 Addressing Scheme

| Device | Interface | IPv6 Address |
| --- | --- | --- |
| R1 | G0/0/0 | 2026::1/64 |
| PC1 | DHCPv6 | Automatically Assigned |

🔹 Step 1: Configure Hostname

enable
configure terminal

hostname R1

Purpose

Assigns a unique name to the router.

🔹 Step 2: Configure Enable Password

enable password 123

Purpose

Protects privileged EXEC mode.

🔹 Step 3: Create Local User

username ccna password 123

Purpose

Creates a local user account for authentication.

| Username | Password |
| --- | --- |
| ccna | 123 |

🔹 Step 4: Configure Domain Name

ip domain-name ccnapracticelab.com

Purpose

Required for SSH key generation and remote management.

🔹 Step 5: Enable SSH Version 2

ip ssh version 2

Purpose

Provides secure remote access to the router.

SSH encrypts communication between client and router.

🔹 Step 6: Enable IPv6 Routing

ipv6 unicast-routing

Purpose

Allows the router to route IPv6 packets.

Without this command:

IPv6 routing will not work.

🔹 Step 7: Configure DHCPv6 Pool

ipv6 dhcp pool LAN1

Create DHCPv6 Pool:

address prefix 2026::/64 lifetime 172800 86400

dns-server 2026::1

domain-name ccnapracticelab.com

Explanation

IPv6 Prefix

2026::/64

Available IPv6 network.

Preferred Lifetime

86400 Seconds

Address remains preferred.

Valid Lifetime

172800 Seconds

Address remains valid.

DNS Server

2026::1

Router acts as DNS reference.

Domain Name

ccnapracticelab.com

Provided to DHCPv6 clients.

🔹 Step 8: Configure Router Interface

Interface:

interface GigabitEthernet0/0/0

Assign IPv6 Address

ipv6 address 2026::1/64

Router IPv6 Address:

2026::1

Enable Managed Configuration Flag

ipv6 nd managed-config-flag

Purpose

Informs clients:

Obtain IPv6 configuration from DHCPv6 Server.

Enable IPv6

ipv6 enable

Attach DHCPv6 Pool

ipv6 dhcp server LAN1

Purpose

Links interface to DHCPv6 Pool.

🔹 Step 9: Configure Console Security

line console 0

password 123

login local

Purpose

Requires username and password when accessing router through console.

🔹 Step 10: Configure VTY Lines

line vty 0 4

login

Purpose

Controls remote access sessions.

🔹 Step 11: Verify IPv6 Interface

show ipv6 interface brief

Expected Output:

GigabitEthernet0/0/0

2026::1

🔹 Step 12: Verify DHCPv6 Pool

show ipv6 dhcp pool

Expected Output:

Pool Name : LAN1

Prefix : 2026::/64

🔹 Step 13: Verify DHCPv6 Bindings

show ipv6 dhcp binding

Displays DHCPv6 clients receiving addresses.

🔹 Step 14: Verify SSH Status

show ip ssh

Expected:

SSH Enabled

Version 2

🔹 Step 15: Test IPv6 Connectivity

From PC:

ipconfig

Expected:

IPv6 Address assigned automatically

Ping Router:

ping 2026::1

Expected Result:

!!!!!

Successful.

🔹 Step 16: Save Configuration

copy running-config startup-config

📚 Verification Commands

show running-config

show ipv6 interface brief

show ipv6 dhcp pool

show ipv6 dhcp binding

show ipv6 route

show ip ssh

ping 2026::1

🎓 Concepts Learned

✅ IPv6 Addressing

✅ IPv6 Unicast Routing

✅ DHCPv6 Configuration

✅ DHCPv6 Address Assignment

✅ IPv6 Prefix Configuration

✅ DNS Server Assignment

✅ Domain Name Assignment

✅ SSH Version 2

✅ Local User Authentication

✅ Console Security

✅ IPv6 Connectivity Testing

📝 Lab Summary

In this lab, a Cisco router was configured for IPv6 networking using the prefix 2026::/64. IPv6 unicast routing was enabled, and a DHCPv6 pool named LAN1 was created to automatically assign IPv6 addresses, DNS information, and the domain name ccnapracticelab.com to clients. Basic router security was implemented through local user authentication, console password protection, and SSH Version 2 configuration. The router interface GigabitEthernet0/0/0 was assigned the IPv6 address 2026::1/64 and connected to the DHCPv6 pool. The lab was successfully completed when the client PC obtained an IPv6 address dynamically and successfully pinged the router's IPv6 address 2026::1, confirming proper DHCPv6 operation and IPv6 connectivity.