🌐 CCNA Lab 04 – Floating Static Routing

📌 Lab Title

Configuration of Floating Static Routes for Network Redundancy

🎯 Lab Objective

The objective of this lab is to configure Floating Static Routing between multiple routers to provide a backup path when the primary route fails.

Floating Static Routes are configured with a higher Administrative Distance (AD) than the primary static route. If the primary route becomes unavailable, the backup route automatically becomes active.

🛠️ Devices Used

| Device | Quantity |
| --- | --- |
| Cisco 1841 Router | 4 |
| Cisco 2960 Switch | 2 |
| PC | 4 |
| Serial DCE/DTE Cables | 4 |
| Copper Straight Through Cables | 6 |

📊 Network Topology

R2

192.168.2.0/30

/      \

/        \

/          \

R1            R4

\          /

\        /

\      /

192.168.3.0/30

R3

Primary Path:

R1 → R2 → R4

Backup Path:

R1 → R3 → R4

📋 IP Addressing Scheme

Router R1

| Interface | IP Address |
| --- | --- |
| Fa0/0 | 192.168.1.1/24 |
| S0/0/0 | 192.168.2.1/30 |
| S0/0/1 | 192.168.3.1/30 |

Router R2

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.2.2/30 |
| S0/0/1 | 192.168.4.1/30 |

Router R3

| Interface | IP Address |
| --- | --- |
| S0/0/0 | 192.168.3.2/30 |
| S0/0/1 | 192.168.5.1/30 |

Router R4

| Interface | IP Address |
| --- | --- |
| Fa0/0 | 192.168.6.1/24 |
| S0/0/0 | 192.168.5.2/30 |
| S0/0/1 | 192.168.4.2/30 |

LAN Devices

Left Side LAN

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 192.168.1.1 |
| PC1 | 192.168.1.20 | 192.168.1.1 |

Right Side LAN

| Device | IP Address | Gateway |
| --- | --- | --- |
| PC2 | 192.168.6.10 | 192.168.6.1 |
| PC3 | 192.168.6.20 | 192.168.6.1 |

🔹 Step 1: Basic Configuration on R1

enable

configure terminal

hostname R1

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

🔹 Step 2: Configure Interfaces on R1

interface fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 192.168.2.1 255.255.255.252

clock rate 64000

no shutdown

exit

interface s0/0/1

ip address 192.168.3.1 255.255.255.252

clock rate 64000

no shutdown

exit

🔹 Step 3: Basic Configuration on R2

enable

configure terminal

hostname R2

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

Configure Interfaces

interface s0/0/0

ip address 192.168.2.2 255.255.255.252

no shutdown

exit

interface s0/0/1

ip address 192.168.4.1 255.255.255.252

no shutdown

exit

🔹 Step 4: Basic Configuration on R3

enable

configure terminal

hostname R3

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

Configure Interfaces

interface s0/0/0

ip address 192.168.3.2 255.255.255.252

no shutdown

exit

interface s0/0/1

ip address 192.168.5.1 255.255.255.252

no shutdown

exit

🔹 Step 5: Basic Configuration on R4

enable

configure terminal

hostname R4

enable secret cisco123

line console 0

password console123

login

exit

service password-encryption

Configure Interfaces

interface fa0/0

ip address 192.168.6.1 255.255.255.0

no shutdown

exit

interface s0/0/0

ip address 192.168.5.2 255.255.255.252

no shutdown

exit

interface s0/0/1

ip address 192.168.4.2 255.255.255.252

no shutdown

exit

🔹 Step 6: Configure Primary Static Routes

On R1

ip route 192.168.6.0 255.255.255.0 192.168.2.2

Explanation

Primary path:

R1 → R2 → R4

On R4

ip route 192.168.1.0 255.255.255.0 192.168.4.1

Primary path:

R4 → R2 → R1

🔹 Step 7: Configure Floating Static Routes

On R1

ip route 192.168.6.0 255.255.255.0 192.168.3.2 10

Explanation

- Administrative Distance = 10

- Used only if primary route fails

Backup Path:

R1 → R3 → R4

On R4

ip route 192.168.1.0 255.255.255.0 192.168.5.1 10

Backup Path:

R4 → R3 → R1

🔹 Step 8: Configure Routes on Transit Routers

R2

ip route 192.168.1.0 255.255.255.0 192.168.2.1

ip route 192.168.6.0 255.255.255.0 192.168.4.2

R3

ip route 192.168.1.0 255.255.255.0 192.168.3.1

ip route 192.168.6.0 255.255.255.0 192.168.5.2

🔹 Step 9: Verify Routing Table

On R1:

show ip route

Output:

S    192.168.6.0/24 [1/0] via 192.168.2.2

Notice that the route with AD 1 is installed.

The floating route with AD 10 remains in the routing table as a backup.

🔹 Step 10: Test Connectivity

From PC0:

ping 192.168.6.10

Expected:

!!!!!

🔹 Step 11: Simulate Link Failure

Shutdown R1's connection to R2:

R1(config)# interface s0/0/0

R1(config-if)# shutdown

🔹 Step 12: Verify Failover

Check routing table again:

show ip route

Output:

S 192.168.6.0/24 [10/0] via 192.168.3.2

Now the floating static route becomes active.

Traffic automatically follows:

R1 → R3 → R4

🔹 Step 13: Verify Connectivity Again

ping 192.168.6.10

Ping should still be successful because the backup route has taken over.

🔹 Step 14: Save Configuration

On all routers:

copy running-config startup-config

🎓 Concepts Learned

✅ Basic Router Configuration

✅ Interface Configuration

✅ Static Routing

✅ Administrative Distance

✅ Floating Static Routing

✅ Route Failover

✅ Backup Paths

✅ Redundant Network Design

✅ Network Troubleshooting

📝 Lab Summary

In this lab, four Cisco routers were configured in a redundant topology. Router R1 and Router R4 connected two separate LANs (192.168.1.0/24 and 192.168.6.0/24). A primary static route was configured through R2, while a backup floating static route was configured through R3 using a higher Administrative Distance of 10. When the primary link was manually shut down, the routers automatically switched to the backup route, ensuring uninterrupted connectivity. This lab demonstrated how Floating Static Routing provides network redundancy and automatic failover in enterprise environments.